Press Information

Strip processing in aluminium rolling mill

Heinrich GEORG Maschinenfabrik:
New high-speed side-trimming line for Elval

Kreuztal, Germany, 8 April 2024     In October 2023, ElvalHalcor Hellenic Copper and Aluminium Industry S. A. commissioned a new high-speed side-trimming line for aluminium strip. Designed to operate at strip speeds of up to 1,650 m/min, the new trimming line is one of the fastest of its kind in the world.

The new line for Elval’s Oinofyta works in Greece was supplied by GEORG on a turnkey basis. The supply range comprised all systems from the payoff reel up to the transfer of the coil to the packaging area. The line is used to side-cut aluminium strip of grades 2XXX to 5XXX rolled on Elval’s new six-high mill. Although most of the material processed is uncoated strip for beverage cans, the line is designed to handle the strip surfaces so smoothly that it can also be used for colour-coated strips. The up to 2,100 mm wide strips can be trimmed at line speeds of up to 1,650 m/min.

The line operates virtually fully automatically: It receives all parameters such as width, thickness and alloy of each coil to be processed from the works MES. Based on these data, the line control system adjusts the cutting depth of the blades and the cutting gap width. By optimizing the processes in all plant areas, very short coil-to-coil times are achieved, enabling annual throughputs of more than 400,000 t.

Arranged behind the payoff reel, an inline drum-type shear prepares the coils for the trimming process. The shear can crop the head and tail ends at strip speeds of up to 200 m/min. Compared to a start-stop solution, inline operation of the shear avoids strip braking and reaccelerating.

The high precision and repetition accuracy with which the side-cutting blade is adjusted achieves a cutting accuracy distinctly superior to that required by the DIN EN 485 standard.

A special feature of the line is the possibility to perform intermediate side trimming if edge cracking is detected during rolling in the six-high mill. In such cases, the affected edge zone can be removed in the trimming line so that rolling of the respective coil can continue as planned.

Due to the exceptionally high line speed, the trimmings are cleaned by a suction system and automatically wound on a scrap winder fitted with a wet scrubber to remove the flakes arising during cutting.

The spraying width of the electrostatic oiling system, a GEORG proprietary development, automatically adjusts to the respective strip width. This avoids excessive oil accumulation on the edge zone and reduces oil consumption. Suction systems arranged along the line ensure that oil droplets coming off the strip surface due to the high line speed are reliably removed, keeping oil emissions extremely low.

In the recoiling area, a manipulator fits and centers the empty sleeves onto the recoiler mandrels. The coil head-ends are automatically bonded to the sleeves. A coil lifting car picks up the finished coils and transfers them to the packaging area.

GEORG supplied the line complete with a wide range of measuring and inspection systems such as an x-ray thickness measuring system, a pinhole detection device and a surface inspection system.

For the GEORG engineers, energy efficiency was an additional important aspect to consider in the line design: During the trimming process, generator technology of the payoff reel generates energy that is fed back into the system. With a high-power system of several hundred kW, substantial energy savings can be achieved in this way.

Detlef Neumann, Sales Manager Finishing Lines at GEORG, commenting on this successful project: “In the global aluminium industry, there is just a handful of trimming lines operating reliably at such high speed. Therefore, we are very happy and proud that our customer has issued the FAC after a very short test run and that the line has been operating highly efficiently ever since.”

GEORG had already supplied several other machines and equipment to Elval before: a drum-type shear for the colour-coating line, two Ultragrind roll grinding machines and a slitting line for up to 2,600-mm-wide strip.

670 words

Figures and captions

Download: press photos GEORG

| Fig. 1: The coils to be side trimmed are produced in the new six-high mill. File name: GEORG_IMG_1173.jpg |  |
| --- | --- |
| Fig. 2a: The entry section viewed from the downcoiler. A wide range of measuring systems assure the high quality of the finished coils. File name: GEORG_IMG_20230524_153637.jpg |  |
| Fig. 2b: The entry section viewed from the downcoiler. A wide range of measuring systems assure the high quality of the finished coils. File name: GEORG_IMG_20230524_153637_a.jpg |  |
| Fig. 3: Control room of the trimming line File name: GEORG_IMG_0949.jpg |  |
| Fig. 4: A perfect coil being recoiled. File name: GEORG_IMG_1150.jpg |  |

Image rights: Heinrich GEORG Maschinenfabrik

About Heinrich GEORG Maschinenfabrik

GEORG specializes in the design of advanced, cost-efficient solutions for coil processing and roll grinding machines. In its solutions, GEORG combines classical mechanical engineering expertise with smart process optimization and service know-how. The company’s cutting-edge finishing lines, world-leading production equipment for the transformer industry and advanced roll grinding machines are in operation worldwide at leading manufacturers of premium products. There, they make a significant contribution to process optimization and the companies’ business success. There, they make a significant contribution to process optimization and the companies’ business success. Machines and systems from GEORG are noted for their high level of automation and used in both highly customized and standard applications. The portfolio is complemented by a broad range of services, including consultation, engineering and maintenance.

The family-owned company, which has more than 500 employees, is now in its third generation. Via its global network of sales and service offices, GEORG supplies its products to key markets, including energy, mobility, and manufacturing. GEORG observes the principles of sustainability, resource saving and energy efficiency.